| Document title | GS318A0001 Technical documents/drawing - Classification of requirements |  |  |
| --- | --- | --- | --- |
| Document Type | Standard | Valid from | 20-March-2023 |
| Author | GMAPFV | Owner | GMALRS |
| If this is a printed version (copy), the original can be found in IMS/QMS/SC via Insite/Toolbox. |  |  |  |

Document history

| Version | Changes from last revision and why |
| --- | --- |
| 13.0 | Reviewers: GMARASV; GMAHEHR; GMALRS; GMATKR; GMAGYF; GMAKBO; GMAVKRV; GCHFRX - General - Moved to new template - Reviewer GMAKEH replaced by GMARASV - Section 4.1.9 Symbol GSI and Section 4.2.6 Example symbol GSI A symbol “GSI” is defined for classification of Grundfos Standard Interfaces. The symbol is only for internal Grundfos information and will be applied on drawings to indicate that the specification is part of a Grundfos Standard Interface, meaning that any change of this IF-specification shall be approved by the interface/module manager. Requested by the Grundfos GTI\A&M department. - Section 4.3 Requirements matrix and overview Exemption on class 0 – adjusted and aligned with GS402A0003 and GS402B0002. To avoid inconsistency regarding Roles and Responsibility these are now referring to GS402A0003. - Section 5.1 Definition of classes and symbols - Column with short definition added. - GSI added. - Section 5.2 definition of other topics - GSI added. - Section 5.3 Abbreviations used in the standard - GTI and A&M added. - Reference - Reference added to GS402A0003 - Handling of Exemptions - Reference added to GS402B0002 - Quality Red Alert (RA) – Escalation process - Reference to SCC workflow removed as this is now part of GI318A0001-02 |

Table of Contents

1	Purpose	3

2	Scope	3

3	Responsibilities	4

3.1	RASCI profile for classification of requirements on Part drawings,  Group Assembly/Group Control Instructions and other specifications owned by Development	4

3.2	RASCI profile for classification of requirements on Control Instructions and assembly instructions owned by AME/ME	5

4	Requirements	6

4.1	Requirement classes definition	6

4.1.1	Class 0 – Regulatory and safety critical requirements	7

4.1.2	Class 2 – Functional and Process critical requirements	10

4.1.3	Class 3P – Process requirements critical for next process step	11

4.1.4	Non-classified requirements	12

4.1.5	Mandatory documentation [D]	13

4.1.6	Least and maximum material limits on geometrical requirements	14

4.1.7	Poka-Yoke	14

4.1.8	Modifier ST	15

4.1.9	Symbol GSI for Grundfos Standard Interface	15

4.2	Examples	16

4.2.1	Examples class 0	16

4.2.2	Examples class 2F; Geometrical requirements	25

4.2.3	Examples Class 2F; LML / MML	28

4.2.4	Examples class 3P	29

4.2.5	Examples modifier ST	31

4.2.6	Examples symbol GSI	32

4.3	Requirement matrix and overview	34

5	Definitions	36

5.1	Definition of classes and symbols	36

5.2	Definition of other topics	36

5.3	Abbreviations used in the standard	37

5.4	HAZARD types and definition	38

6	Records	39

7	References	39

8	Appendix	40

8.1	Class 0 requirements on Control Instructions	40

8.2	Legacy definition of Class [1]	41

8.3	Application of GS 318A0001 to HW documents	42

8.4	Converting requirements and requirements fulfilled by the chosen process	43

8.5	Note – double meaning of the class 0 symbol: ISO 8062-2 vs. GS 318A0001	44

Purpose

This Group Standard specifies rules for the classification of critical requirements on drawings, documents and in other specifications.

The purpose of the classification system is to indicate requirements of special importance. This document describes what classified requirements are, why they are important, how to classify the requirements. How to secure classified requirements is often a series of quality assurance activities described in various standards such as:

- Control Plans (GS402A0043)

- Control Instructions (GS401A0020 - only used internally Grundfos)

- VPC - Verification of Product & Process Conformance (GS402A0006)

- Capability studies (GS402A0046)

Scope

The classification system is applicable to purchased raw materials, components, and products as well as to Grundfos’ own processes, components, products, and systems.

The Standard shall be applied on specifications related to MECH and PRODUCT/SYSTEM processes, and to some extend also HW processes (See Section 8.3)

The classification system shall be applied for all types of critical requirements, for example but not limited to:

- Geometrical tolerances

- Mechanical requirements

- Surface requirements

- Product Routine Test requirements

- Material composition, type, grade, name, or supplier

Responsibilities

The responsibility for classifying requirements is defined by the two RASCI profiles:

- RASCI for documents owned by Development

- RASCI for documents owned by AME/ME

RASCI profile for classification of requirements on Part drawings, 
Group Assembly/Group Control Instructions and other specifications owned by Development

| RASCI profile for classification of requirements on Part drawing, Group Assembly/Group Control Instruction and other specifications from Development |  |  |  |
| --- | --- | --- | --- |
|  | Classification of requirements on new drawings/documents | Decide if a Safety Critical Requirement shall have mandatory Documentation (D) or not. (Use GI 318A0001-02 for evaluation of tolerance chains) | Ensuring suppliers receive classification to the requirements on sourced components/products |
| Responsible | Product Development: The development Engineer creating/updating the drawing (SAP Document owner role) | Product Development: The development Engineer creating/updating the drawing (SAP Document owner role) | Group Purchasing, Supplier Quality |
| Accountable | Product Development: Drawing/document approver (SAP document approver role) | Product Development: Drawing/document approver (SAP document approver role) | Group Purchasing |
| Supporting | AME (Note1) | Group Quality | Supplier Quality, Development |
| Consulted | Product Compliance (Note 2), Operations | Product Compliance, | Operations |
| Informed | Group Purchasing, Group Quality, Operations | - | Suppliers |
| Note 1: Especially with respect to definition of [2P] requirements. Note 2: Consult Product Compliance regarding all class 0 requirements. |  |  |  |

RASCI profile for classification of requirements on Control Instructions and assembly instructions owned by AME/ME

| RASCI profile for classification of requirements on Control and Assembly Instructions |  |  |  |
| --- | --- | --- | --- |
|  | Lifecycle status of the Product |  |  |
|  | Until approved 0-meeting | Approved 0-meeting – DP7 | After DP7 |
|  | Classify requirements | Keep classification updated | Keep classification updated |
| Responsible | Advanced Manufacturing Engineering (AME) (Note 1), Development (Note 2) | Manufacturing Engineering (ME) (Note 3) | Manufacturing Engineering (ME) (Note 4) |
| Accountable | Manufacturing Product Manager (MPM) | Manufacturing Engineering Product Manager (MEPM) | Manufacturing Engineering (ME) -department head |
| Supporting | Advanced Manufacturing Engineering (AME) | Advanced Manufacturing Engineering (AME) | Advanced Manufacturing Engineering (AME) |
| Consulted | Operations/Production | Operations/Production, Development | Operations/Production, Development |
| Informed | Production | Production | Production |
| Note 1) Manufacturing (AME/ME) has the responsibility to prepare a draft on how the Control/Assembly Instruction shall look like. This will usually be made based on requirements (incl. classification) on the Part/Assembly drawings and in close cooperation with ME. Note 2) Based on the draft from AME, development has the responsibility to create the first version of the needed control/assembly instructions in Catia and update it based on the 0-meeting + release in SAP. These Control instructions shall be with text in English. Note 3) After the 0-meeting, the Control Instruction is released in SAP and the local ME department updates the Control Instruction by adding local language if needed and keeps the classification updated. Note 4) ME in the production site has the responsibility to keep the classification updated and to decide if requirements on old instructions shall be classified. |  |  |  |

Requirements

The classification system shall be applied in connection with requirements on all drawings, in test specifications, master records, control instructions and other specifications. The symbols shall be placed next to the specified requirements.

Requirement classes definition

There are 3 classes which are defined in the table below:

| Class | Definition | Details in |
| --- | --- | --- |
| 0 | Regulatory requirements and/or safety critical requirements with 3rd part approval or self-declaration. Includes: - Safety critical requirements - Regulatory but not safety critical requirements | Section 4.1.1 |
| 1 | Regulatory requirement without product approval including safety where self-declaration is allowed. This requirement class has been phased out and is only used on documents released before 1-1-2019 | Section 8.2 |
| 2 | Functional and process critical requirements (2=2F+2P) | Section 4.1.2 |
| 2F | Functional critical requirements | Section 4.1.2 |
| 2P | Process critical requirements. Requirement is critical for one or more of the following process steps | Section 4.1.2 |
| 3P | Process requirements without capability. Requirement is only critical for the next process step. | Section 4.1.3 |

Class 0 – Regulatory and safety critical requirements

| Symbol | Class | Definition |
| --- | --- | --- |
|  | 0 | Class 0 symbol or (scale 2) placed next to description in the drawing head Safety/regulatory critical part (Internal Grundfos information) Meaning that the Grundfos Product Development shall look up the reason for the classification before making any changes (*) to the part. Consult SAP CERTIFICATIONS class, D-FMEA, Design Journal and/or the Design Verification to find reason and information for the marking. If question comes up regarding compliance, then Product Compliance shall be consulted to clarify if the change can be implemented *) changes to ratings, designation, composition, geometry, tolerances, or supplier of the part |
|  | 0 | Class 0 symbol with [D] and (hazard) identification shall be used for safety critical requirements. These requirements are typically derived from regulatory requirements. Hazard types acc. to Section 5.4. Mandatory documentation acc. to section 4.1.5. No changes are allowed on ratings/designation/composition/geometry/tolerances/ supplier of the part without prior acceptance from Grundfos Product Compliance. |
|  | 0 | Same as above just without “mandatory documentation”, see section 4.1.5 No changes are allowed on ratings/designation/composition/geometry/tolerances/ supplier of the part without prior acceptance from Grundfos Product Compliance. |
|  | 0 | Class 0 without [D] and without hazard identification shall be used for: - Regulatory but not safety critical requirements with 3rd part approval or self-declaration Example: MEI > 0,52 (requirement in a Routine Test Specification to ensure that we fulfil marked MEI on name plate. - characteristics which are copied into 3rd part approval files used to document what is tested and approved. (Because a change shall also be reflected in the 3rd part approval file) Example: Stator diameter and length of stator. |
| [D](Thermal/fire) | 0 | Class 0 symbol with [D] and (hazard) identification shall be used for safety critical requirements on Ex products. These requirements are typically derived from regulatory requirements. Hazard on Ex is by default “Thermal/fire” acc. to Section 5.4. Mandatory documentation acc. to section 4.1.54.1.4. No changes are allowed on ratings/designation/composition/geometry/tolerances/ supplier of the part without prior acceptance from Grundfos Product Compliance. Example: Flame gap |
| (Thermal/fire) | 0 | Same as above just without “mandatory documentation”, see section 4.1.54.1.4 No changes are allowed on ratings/designation/composition/geometry/tolerances/ supplier of the part without prior acceptance from Grundfos Product Compliance. Example: dimension with large margin to the safety critical requirement (see Figure 13) |
|  | 0 | Class 0 symbol without [D] and hazard identification shall be used for regulatory but not safety critical requirements on Ex products: - Regulatory requirements with 3rd part approval or self-declaration - Characteristics which are copied into 3rd part approval files used to document what is tested and approved. (Because a change shall also be reflected in the 3rd part approval file) Example: Overall dimension of the Ex-part |

- Safety critical requirements and regulatory requirements:

- Documents created before 1-1-2019
Regulatory requirements with third part approval. The classification shall be applied for all products holding a 3rd part product approval, and where the specifications are a necessity/critical to this product approval. Examples: UL approval, ATEX approval, CSA approval.

- Documents created after 1-1-2019: 
Regulatory requirements independent if it is self-declaration or third part approvals. The classification shall be applied for all products holding a 3rd part product approval and products with self-declaration, where the specifications are a necessity/critical to fulfil these regulatory requirements. 
Examples: UL approval, ATEX approval, Low Voltage Directive.

- The requirements shall be stated on drawings and when necessary, in related documents, e.g. drawings of raw parts, control instructions, etc. and/or specified in SAP classification data.

- For safety critical requirements the Hazard type acc. to Section 5.4 shall be indicated after the classification in brackets, See examples in Section 4.2.1. Except for the symbol  as this is by default hazard type “Thermal/fire”

- All components/elements/parts and products which has one or more class 0 requirements shall be marked in SAP certification class (MECH, HW and PRODUCT/SYSTEM).

- Classification shall be applied to lowest possible level

- For Grundfos in-house manufactured parts: Section 8.1 describes how class 0 requirements on Part/Assembly Drawings shall be transferred to the Control Instruction.

- Instructions
Level where the Safety classification is applied
It is recommended to classify the safety critical requirement on the level where it can be controlled. Example: Clearance and creepage on stator, see example Figure 3.

But sometimes it is an economic advantage to bundle more sub-safety critical requirements into one higher level safety requirement. Example: The thermal hazard on a pump caused by high pressure fluid ejection/leakage of hot liquid would normally be cheaper to control as a high-pressure test as part of the routine test instead of specifying requirements to material strength, porosities, thickness, fillets etc. on casted level of all the pressure critical components in a pump, see Figure 11

- The class 0 symbol  or   shall be applied to the drawing head for drawings which are regulatory and/or safety critical, see Figure 1 and Figure 2.

- See examples in Section 4.2.1

- Apply the internal Grundfos Instructions:

  - GI318A0001-01
Classification of drinking water critical requirements.

  - GI318A0001-02
Evaluation of tolerance chains to minimize the need for safety classification.

  - GI318A0001-03
Documentation and classification of critical components in SAP CERTIFICATIONS class.

Figure 1:  The default stickers and symbols to be applied on document with class 0 Safety Critical Requirements except if it is an Ex-approved product. Apply the class 0 symbol in the drawing head description in scale 2 to clearly communicate that the drawing includes class 0 requirement(s).

Figure 2:  The default stickers and symbols to be applied on document with class 0 Safety Critical Requirements for Ex-approved product. Apply the class 0 symbol in the drawing head description in scale 2 to clearly communicate that the drawing includes class 0 requirement(s).

Interpretation

The supplier's or Grundfos' own production equipment and processes together with the accompanying quality assurance activities shall ensure that the requirements for process capability acc. to GS402A0046 are always observed. For Grundfos internal manufacturing this implies that Control Limits acc. to GS402A0050 shall be applied.

The certification class in SAP shall be updated with the information on which approvals apply to the part and the technical file no. shall be listed.

Class 2 – Functional and Process critical requirements

| Symbol | Class | Definition |
| --- | --- | --- |
| [2F] | 2 | Functional critical requirement related to functional Critical Characteristic (CC). To be used when the requirement is of critical importance to the function of the finished part/product. Change of the specification or exemption require acceptance from the technical responsible engineer in Product Development |
| [2P] | 2 | Process critical requirements related to process Critical Characteristic (CC) . Requirement is critical for one or more of the following process steps Change of the specification or exemption require acceptance from the ME engineer responsible for one or more of the proceeding operational steps which are depending on the 2P classified requirement |
| [2] | 2 | Functional and process critical requirements related to Critical Characteristic (CC) Change of the specification or exemption require acceptance from the technical responsible engineer in Product Development and the ME engineer |

To be used for critical requirements which mean that exceeding tolerance limits will result in risk of product nonfunction, disrupted product function, or makes further production impossible or deteriorated.

Examples of functional critical requirements:

- Requirements which are part of a critical statistical tolerance chain

- Requirements which have major impact to performance characteristics like flow, head, efficiency, noise and vibration

- Requirements where the function loss is significant when tolerance limits are exceeded

- See examples Section 4.2.2 and 4.2.3

Examples of process critical requirements:

- Requirements that are used to control the process (distances, hole patterns, surfaces etc)

Interpretation

The supplier's or Grundfos' own production equipment and processes together with the accompanying

quality assurance activities shall ensure that the requirements for process capability acc. to GS402A0046

are always observed. This implies that Control Limits acc. to GS402A0050 shall be applied.

Class 3P – Process requirements critical for next process step

| Symbol | Class | Definition |
| --- | --- | --- |
| [3P] | 3 | Process requirements without capability Requirement is only critical for the next process step. Change of the specification or exemption require acceptance from by the responsible for the next process step. Suppliers do not need to ask Grundfos if they do the proceeding process themselves (example: Machine a raw casted part). |

Requirements which are only of importance to the next process step and where there is no requirement for capability.

Examples of class 3P requirements:

- Machining allowance

- See examples in Section 4.2.4

Interpretation

| Classification | If the supplier also supplies the next process step on the part | If Grundfos or another supplier performs the next process step on the part |
| --- | --- | --- |
| [3P] “Process requirement critical for the next process step” | The supplier can deviate on the requirement without consulting Grundfos if the next process step is fulfilled. | Deviation requires accept by Grundfos or the other supplier performing the next process step. Either exemption or changed drawing requirement |

Non-classified requirements

| Symbol | Class | Definition |
| --- | --- | --- |
| none | none | Requirements which are not classified Non-classified requirements are those considered less important than the classified. But independent if a requirement is classified or not it shall be fulfilled – meaning that components/parts/products that do not comply on all requirements should either be scrapped or used with an exemption. Non-classified requirements mean that - there is no requirement for capability acc. to GS402A0046 - the VPC (Verification of Product & Process Conformance) would typically be performed on a few samples acc. to GS402A0006 - the requirement shall be verified |

Mandatory documentation [D]

| Symbol | Class | Definition |
| --- | --- | --- |
| [-D] | 0 and 2 | Mandatory documentation meaning that at any time during the given filing period, it shall be possible by the aid of records to demonstrate that the requirements were fulfilled at the time of production. Class 0 Mandatory Documentation shall be applied to all Safety Critical Requirements (to show evidence of conformance which is essential if alert issues occur) except - if the design is made with a large margin to the safety critical limit (which is not necessarily the same as the tolerance limits), see section 3.1 regarding responsibility. - for design Poka-Yoke’s (because this is documented in Product Development). Mandatory Documentation can be applied to the regulatory requirements in class 0. Class 2 Mandatory Documentation can be applied to the requirements in class 2. |

| Documentation method for requirements with “Mandatory Documentation” |  |
| --- | --- |
| Documentation | - If the authority/3rd part requires a specific documentation method this shall be specified on the drawing. Otherwise, either sample control or 100% control shall be applied for monitoring and documentation. The chosen method shall be documented in the Control Plan*and finally agreed upon on the 0-meeting. |
| Sample control | - Process capability shall comply with GS402A0046 - The [D]-classified requirements shall be measured according to the sampling strategy from the Control Plan* |
| 100% control | - No requirement for capability acc. to GS402A0046 - The [-D]-classified requirements shall be measured on all parts manufactured acc. to the Control Plan* |
| Collection of measured data: | - Variable measurements: Collect all measured data in the batch ** - Attribute measurements: Collect number of OK, NOT OK and TOTAL number (OK+NOT OK) of produced parts in the batch ** |
| Storage of data | - The measured data shall be collected and stored so it is possible at any point of time in the filing period to show the data on request |
| Filing period | Classification; Minimum filing period of collected measured data after manufacturing date (if not stated otherwise on the drawing). [D] (Thermal/fire); 10 years. [D] (hazard); 5 years (10 years in regards of Fire pumps). [2FD], [2PD] or [2D]; 5 years. |
| Traceability | If a traceability is required between the specific DUT and the measurement, this shall be specified on the drawing including how the DUT ID shall be made. |
| * For Grundfos in-house manufactured parts: If a Control Plan does not exist then use the sampling specification in the Control Instruction. ** A batch can also be a production period like a day or a week. Definition of batch to be stated in the Control Plan. |  |

Least and maximum material limits on geometrical requirements

| Symbol | Class | Definition |
| --- | --- | --- |
| [*_MML] | 0,2,3 | The classification only applies to the MML (Maximum Material Limit) |
| [*_LML] | 0,2,3 | The classification only applies to the LML (Least Material Limit) |

- On geometrical Product Specifications of the types, position, form of surface, and linear size, MML or LML can be applied

- If [*_MML] or [*_LML] is applied in class 0, 2 or 3, the symbols can be combined, e.g. [2F_MML]; [2FD_MML]; [2_LML]

- See example Figure 9

Poka-Yoke

| Symbol | Class | Definition |
| --- | --- | --- |
|  | 0,2,3 | The feature is defined as a poka-Yoke mistake proof mechanism. A Poka-yoke can either be a design or a process Poka-Yoke: Design Poka-yoke A Design Poka-yoke is a feature in the product design. - Classification: Classify safety and functional critical Design Poka-Yoke solutions to ensure that it is not modified / removed unintentionally - Example: A feature in a cable plug ensuring that the plug cannot be assembled incorrect. See example Figure 8 - Verification: To be verified and documented in Product Development. Process Poka-yoke A Process Poka-yoke is a feature in the process, tool, equipment or fixture. - Example: Assembly fixture ensuring correct position of stator and power supply cables. - Verification: Refer to GS401A0005 |

Modifier ST

| Symbol | Class | Definition |
| --- | --- | --- |
|  | 0 and 2F | The modifier ST is used to specify requirements for the population: The ST shall only be used for specific tolerances which are very sensitive to the population in safety/functional critical statistical tolerance chain and thereby compromises the function of the product. If the specific part with ST mark is being sorted, then reduced sorting limits of 60% the tolerance shall be used. A sticker explaining the modifier ST and the sorting limits shall be applied to the drawing: Ref. ISO 18391 – GPS population specification |

See examples Section 4.2.5

Symbol GSI for Grundfos Standard Interface

| Symbol | Class | Definition |
| --- | --- | --- |
|  | Independent of classes | The symbol “GSI” is an internal Grundfos information used to indicate that the specification is part of a Grundfos Standard Interface. The GSI-symbol shall be applied on: - Grundfos Standard Interface document The GSI classification shall be applied to the interface specification on the Grundfos Standard Interface document meaning that any change of the interface specification shall be approved by the module manager appointed by Grundfos GTI\A&M department. See example Figure 28 (Section 4.2.6) - Part Drawings On the Part Drawings, the GSI symbol shall be applied to the Grundfos Standard Interface specification to indicate that the interfaces comply to a Grundfos Standard Interface document. The Grundfos Standard Interface document shall be object linked to the material number (in SAP). See example Figure 29 (Section 4.2.6) |

Examples

As a main rule, the symbols shall be located next to the requirements.

In general, the examples shown in this section are not complete specifications – just examples, i.e. the drawing is not complete.

Examples class 0

| Example | Explanation |
| --- | --- |
| [D] | Class 0 explosion proof products symbol Look in the SAP certification class to see which standard it relates to (could for example be related to ATEX, IECEx, EN50018 with a specific file no which shall be specified in the certification class). Documentation is mandatory. |
| [D] | Class 0 symbol. Look in the SAP certification class to see which standard it relates to (could for example be related to UL 778 with a specific file no which shall be specified in the certification class). Documentation is mandatory. |
| (In SAP classification) Strength class: “12.9 Class 0 (Thermal/fire) GS 318A0001” | Class 0 safety critical requirement. Example on classification of a standard component which does not have a Grundfos Part Drawing: This example is for a M16x40 screw where the strength class 12.9 is a safety critical requirement related to the hazard type “Thermal/fire”. In SAP class the class 0 requirement is specified together with the hazard type and a reference to GS318A0001. |

Figure 3: Example with 4 class 0 classified requirements: 2 requirements (green) which are class 0 because these dimensions are copied into the 3rd part approval file (e.g., UL file) and 2 requirements (red) which are safety critical with the hazard Electric Shock. Furthermore, the class 0 sticker is applied to drawing head to indicate that it is a Safety/regulatory critical part, and no changes are allowed on ratings, designation, composition, geometry, tolerances or supplier of the part without revisit of the D-FMEA and Design Verification in general.

Figure 4: Example showing a drinking water critical seal ring made by carbon graphite with the specific trade name “CY103” from our supplier Morgan. The class 0 symbol scale 2 next to the description in the drawing header means that it is a safety critical part, and no changes are allowed on ratings, designation, composition, geometry, tolerances, or supplier of the part without revisit of the design documentation, and in this case, consult drinking water compliance specialist. 
The related Drinking Water Specification Sheet is shown in Figure 5

Figure 5: Example showing a Drinking Water Specification Sheet (DWSS) for the carbon-graphite raw material trade name “CY103” from our supplier Morgan. All requirements in the DWSS are safety critical with the hazard contamination referring to drinking water. The DWSS is object linked to the seal ring shown in Figure 4.

Figure 6: Example showing a plug for stator where the material is safety critical with the hazard “Electric shock”, and with mandatory documentation that the material must be ETL and VDE listed. The 2-component material designation is also classified safety critical with the hazard “Electric shock”. Furthermore, the class 0 sticker is applied to drawing head to indicate that it is a Safety/regulatory critical part, and no changes are allowed on ratings, designation, composition, geometry, tolerances or supplier of the part without revisit of the D-FMEA and Design Verification in general.

Figure 7: Example showing classification class 0 with documentation (D) with Hazard “Electric shock” on a requirement regarding electrical safety acc. to IEC 60355-1 and UL1004-1 (clearance). On the drawing the general class 0 sticker safety critical parts is applied, and the class 0 symbol + D is applied to the two times min. 2,5mm. Extract from certification class is also shown with yes to UL critical part and the UL file no can also be found. Furthermore, the class 0 sticker is applied to drawing head to indicate that it is a Safety/regulatory critical part, and no changes are allowed on ratings, designation, composition, geometry, tolerances or supplier of the part without revisit of the D-FMEA and Design Verification in general.

Figure 8: Example of a Design Poka-Yoke feature on a plug defined on a part drawing and classified as class 0 safety critical requirement. The Poka-Yoke feature is related to the Hazard “Electric shock” and ensures that the PE/grounding and power connection to the pumps is correct and safe. Although it is a Safety Critical Characteristic it is not with documentation (D) because there is a large margin to the safety critical limit. Furthermore, the class 0 sticker is applied to drawing head to indicate that it is a Safety/regulatory critical part, and no changes are allowed on ratings, designation, composition, geometry, tolerances or supplier of the part without revisit of the D-FMEA and Design Verification in general.

Figure 9: Example where the thickness of the two ribs is functional critical [2F_MML] – to ensure sufficient strength and space for assembly. The thickness is Safety Critical in the Least Material Limits to fulfil requirement for minimum material thickness to avoid electric shock. The rib with thickness 2,5mm is designed with a large margin to the requirement and is thus without mandatory documentation. Furthermore, the class 0 sticker is applied to drawing head to indicate that it is a Safety/regulatory critical part, and no changes are allowed on ratings, designation, composition, geometry, tolerances, or supplier of the part without revisit of the D-FMEA and Design Verification in general.

Figure 10: Example of a part which is safety critical class 0 with the Hazard type “Electric shock”. On the drawing the general class 0 sticker safety critical part is applied, and the class 0 symbol is applied to a geometrical dimension and to the volume. The total number of class 0 requirements is 9 because there is 8x 2.68±0.07 together with the requirement to volume 3.022±75cm3. Furthermore, the class 0 sticker is applied to drawing head to indicate that it is a Safety/regulatory critical part, and no changes are allowed on ratings, designation, composition, geometry, tolerances, or supplier of the part without revisit of the D-FMEA and Design Verification in general.

Figure 11:  Example of Product Routine Test Specification where the safety critical pressure test of hazard type “Thermal/fire” is classified 0 with mandatory documentation. Hazard type “Thermal/fire” correspond to water above 60°C.  The Pressure test is acc. to EN809 and EN12162 used as design standards and for CE marking with self-declaration. Furthermore, the class 0 sticker is applied to drawing head to indicate that it is a Safety/regulatory critical part, and no changes are allowed on ratings, designation, composition, geometry, tolerances, or supplier of the part without revisit of the D-FMEA and Design Verification in general.

Figure 12: Example showing an assembly drawing/instruction of a circulator pump with 2 Class 0 requirements of the Hazard types “Electric shock” and one of hazard type “Thermal/fire”. Furthermore, the class 0 sticker is applied to drawing head to indicate that it is a Safety/regulatory critical part, and no changes are allowed on ratings, designation, composition, geometry, tolerances, or supplier of the part without revisit of the D-FMEA and Design Verification in general. In the example the torque values have been classified safety critical under the assumption that double value on the torque tolerances will compromise product safety (refer to GI 318A0001-02 for evaluation of sensitivity of tolerance chains). In the example the assumption is furthermore that cables which are not inserted fully does not compromise product safety.

Figure 13: Detail from part drawing of a rubber plug for an Ex-approved product. The thickness 25,5 +1/-0 is listed in the Schedule drawing, but because the limit in the Ex-standard says min 5mm there is no need to require mandatory documentation [D].
The same goes with the inner and outer diameters which are classified Ex because the nominal dimension is listed in the Schedule drawing in order specify the allowable configurations between cable to rubber plug – also here it makes no sense to require mandatory Documentation [D]. Furthermore, the class 0 sticker is applied to drawing head to indicate that it is a Safety/regulatory critical part, and no changes are allowed on ratings, designation, composition, geometry, tolerances, or supplier of the part without revisit of the D-FMEA and Design Verification in general.

Figure 14: Example showing part of a drawing with many (in this example 7) safety critical requirements each with the same hazard. In such situations it is beneficial to make the reference to the hazard as a note above the drawing header to replace having the hazard next to all the SCC. Furthermore, the class 0 sticker is applied to drawing head to indicate that it is a Safety/regulatory critical part, and no changes are allowed on ratings, designation, composition, geometry, tolerances, or supplier of the part without revisit of the D-FMEA and Design Verification in general.

Figure 15: Example showing a Part Drawing (left) with 2 safety critical class 0 requirements (yellow) and one class 0 non-safety requirement (green)together with the corresponding Control Instruction. 
On the Control Instruction (right) the two safety critical requirements (yellow) are shown with class 0 symbol and a control method + samples strategy is defined. The general class 0 sticker and the hazards are not shown. The scale 2 class 0 symbol is also applied to the CI drawing header. In the example the non-safety requirement is not to be controlled as this is only used to document what is tested and approved in the 3rd part approval file. For more details refer to Section 8.1.

Figure 16: Example of specification with 3 safety critical class 0 requirement: Group Assembly Instruction for XLCR where the quick guide for safe lifting/handling of the product and guards to protect rotational parts acc. to EN809 has been classified as class 0. It was decided that there is no need for documentation (D) for these safety critical characteristics. Hazard and general SCC sticker are not applied as it is a Group Assembly Instruction.

Examples class 2F; Geometrical requirements

For geometrical requirements, the classification symbol can be placed in different places depending on the intention.

The classification shall be placed after the tolerance and the corresponding modifier (GS310A0010), see Figure 17.

Figure 17: Classification of a diameter placed after the tolerance and modifier. Documentation is not mandatory

Geometrical tolerances

The classification can be placed either above the tolerance zone indicator, or after the tolerance in tolerance zone indicator compartment two, see examples in Figure 18.

Figure 18: Classification on geometrical tolerance. a) Classification placed above the tolerance zone indicator. b) Classification placed in tolerance zone indicator compartment two after the tolerance. Documentation is not mandatory.

Stacked geometrical tolerance zones ("GPS stacking")

If the classification applies to all requirements, it shall be placed on the top of the GPS stack, see Figure 19.

If the classification applies to some of the requirements only, or if the classification is different, it shall be placed in the tolerance zone indicator compartment two after the tolerance, see Figure 20.

Figure 19:  The entire GPS stacking classified as critical due to function: 2 surfaces (2x) with 6 requirements, (12 requirements in total): - 2 x Ra roughness, 2 x Rt roughness, 2 x flatness, 2 x NC (not convex), 2 x perpendicularity, and 2 x position. Documentation is not mandatory.

Figure 20: Classification [2F] applies to some requirements only in the GPS stacking: 2 surfaces (2x) each with 3 requirements (6 requirements in total): 2 x Rt roughness, 2 x perpendicularity, and 2 x position. Documentation is not mandatory.

GPS stacking and linear dimension

For GPS stacking combined with linear dimensions where all requirements shall have same classification, the optimum solution is to gather all requirements in a GPS stack, see Figure 21 a).

Unfortunately, Catia does not fully support this solution as the dimension cannot be linked to the 3D geometry. Two work arounds are shown in Figure 21 b) and c).

Figure 21: a) Future solution with a GPS stack including the linear size.
b) Work around 1: Linear dimension and geometrical tolerance positioned on the same side, and the classification above the tolerance zone indicator applies to both the geometrical and the linear requirement.  
c) If the linear dimension and geometrical tolerance are positioned on different sides, both need to be classified.

Examples Class 2F; LML / MML

The symbol may be placed either above the tolerance zone indicator or in the tolerance zone indicator compartment two.

For GPS stacking the symbol shall be placed in the individual compartments.

Figure 22 shows examples where MML and LML have been applied to geometrical requirements.

Figure 22: Examples of classification of geometrical tolerance zones and linear sizes. a + b) Only Least Material Limits are classified as critical due to function.  c) Only Maximum Material Limit is classified as critical due to function. d) Only Maximum Material Limits are classified as critical due to function with documentation obligation.

Figure 23 shows an example where MML has been added to the classification of the diameter of a hole and shaft.

Figure 23: Example where only the Maximum Material Limits are classified as critical due to function, i.e., 
a) Ø25,0 GX (maximum inscribed cylinder on complete feature, and b) Ø20 GN (minimum circumscribed on complete feature.

Examples class 3P

Casted part
Figure 24 shows an example of a casted part where the machining allowance on the raw casted part is classified [3P]:
Grundfos specifies casted parts in 2-3 levels each having individual P/N and drawings: Raw casting, machined part and sometimes also painted/CED coated. 
The machining allowance on the raw casted parts can be classified [3P]:

- On casted parts where the foundry is also machining the part, Grundfos allows that the foundry controls requirements like machining allowance: The foundry may deviate from the [3P] requirement if the requirements on the next process step, which is machined level, are fulfilled. This decision is made to save cost and to give the supplier as high amount of freedom as possible.

- On the other hand, if Grundfos sources the casting and does the next process step, which is machining, internally, then we need to have the machining allowance in control. Deviation from the [3P] requirement shall be approved by Grundfos. See example Figure 24

| Interpretation of classification on raw casted parts |  |  |
| --- | --- | --- |
| Classification | If foundry machine the part | If Grundfos machines the part |
| [3P] “Process requirement critical for next process step” | Foundry may deviate on the requirement without consulting Grundfos | Deviation requires accept by Grundfos. Either exemption or changed drawing requirement |

Figure 24: Example of a [3P] classification on requirement controlling machining allowance on a casted part.

Deep drawn part

The class [3P] can also be used on other processed parts like a deep drawn part which is machined on the next process step, see example Figure 25.

Figure 25: Example of a deep drawn cup where the height is classified [3P].

Examples modifier ST

Figure 26: Example showing how the modifier “ST” with a text note is used to set special requirement if parts are sorted. 
Then the sorting limits are |Pos|0.03|A| - because 60% of the tolerance 0.05 is 0.03.

Figure 27: Example showing how the modifier “ST” with a text note is used to set special requirement if parts are sorted. In this example the “ST” only applies to MML as it is the contact between rotor and stator that is critical in the tolerance chain.  Meaning that the sorting limits are ø74,3 +0,018/-0,03 - because 60% of the MML tolerance 0.03 is 0.18.

Examples symbol GSI

Figure 28: Example showing how the GSI symbol is used to classify requirements on a Grundfos Standard Interface Document for a loose flange system. The symbol is only for internal Grundfos information to indicate that the interface is a Grundfos Standard Interface, meaning that any change of the IF-specification shall be approved by the interface/module manager.

Figure 29: Example showing a Part Drawing with a number of Loose Flange variants where the interface to the pump is in accordance with a Grundfos Standard Interface document shown in Figure 28. These requirements are marked with the GSI symbol.

Requirement matrix and overview

The table below establish a survey on the different classification classes and the relation to various subjects of interest:

|  | Class 0 |  | Class 2F | Class 2P | Class 3P | Non-classified |
| --- | --- | --- | --- | --- | --- | --- |
| Definition Reference | Regulatory requirements and/or safety critical requirements with 3rd part approval or self-declaration |  | Requirement is of critical importance to the function. | Requirement is of critical importance to the current or one of the subsequent production/assembly processes. | Requirement is only critical for the next process step. | Requirement is not classified meaning - it is less important than the classified - but shall still be fulfilled |
|  | Section 4.1.1 |  | Section 4.1.2 | Section 4.1.2 | Section 4.1.3 | Section 4.1.4 |
| Is exemption allowed? Acc. to GS 402A0003 and GS 402B0002 | Yes, temporary, but shall be avoided as far as possible. Requirements, Roles, and Responsibility acc. to GS402A0003. |  | Yes, temporary. Requirements, Roles, and Responsibility acc. to GS 402A0003. |  |  |  |
|  | Safety critical requirements (SCC): Deviations to be handled as Red Alert and the RA team decide if an exemption can be granted (GS402B0002). | Regulatory but not safety critical requirements: PD shall in advance ensure acceptance from the Product Compliance Specialist who shall involve 3rd part agency if needed. |  |  |  |  |
| Proof of compliance GS 402A0046, GS 402A0006 | - Capability (GS402A0046) - Control limits (GS402A0050) or 100% inspection (GS401A0005, see Note 2) - VPC (GS402A0006) |  |  |  | + No requirement for capability + VPC (GS402A0006) (typically on a few samples) | + No requirement for Capability ** + VPC (GS402A0006) (typically on a few samples) |
| 100% manual visual control allowed? | Yes (Note 1) |  | Yes | Yes | Yes | Yes |
| MSA acc. to GS 402A0068 | Mandatory |  | Mandatory | Mandatory | Optional | Optional |
| Control limits on variable data acc. to GS 402A0050 | Mandatory |  | Mandatory | Mandatory | Optional | Optional |
| Mandatory Documentation [D] | Default yes, but with large margin to the safety critical limit the D may be left out, see section 4.1.5 |  | Optional | Optional | No | No |
| d-FMEA score acc. to GS 402A00048 | Severity rating 9+10 |  | Not defined | Not defined | N/A | N/A |
| SRA Index acc. to GF 402A0008-01 | 3-6 |  | 1-2 | 1-2 | N/A | N/A |

Note 1: 100% manual visual inspection should be minimized to largest possible extend as the detection method does not catch all defects. However, we have a few cases like wirings in a motor where manual visual inspection is the only possibility/best solution and in these cases, it requires

- A picture master provided and maintained with examples of ok and the not ok’s. Responsible: Development

- A work instruction provided and maintained. Responsible: ME

- Training/skills shall be defined, and operator needs to be certified to perform the inspection. Responsible: ME

Note 2: For Poka-Yoke there is no requirement for capability, or 100% measurements refer to GS401A0005. In case the <ST> classification is used the specification limits for the Poka-Yoke, or 100% measurement shall be reduced to 60%.

Definitions

Definition of classes and symbols

| Symbol | Class | Short definition | Detailed definition |
| --- | --- | --- | --- |
|  | 0 | Safety or Regulatory critical | Section 4.1.1 |
|  | 0 | Explosive Environment | Section 4.1.1 |
| (hazard) | 0 | Hazard added to a class 0 symbol for a Safety Critical Requirement. | Section 5.4 |
| [1] | 1 | Old symbol. Phased out 2018. | Section 8.2 |
| [2] | 2 | Functional and process critical | Section 4.1.2 |
| [2F] | 2 | Functional critical | Section 4.1.2 |
| [2P] | 2 | Process critical | Section 4.1.2 |
| [3P] | 3 | Process requirement critical for next process step | Section 4.1.3 |
| [-D] | 0,2 | Mandatory documentation | Section 4.1.5 |
| [*_MML] | 0,2,3 | Maximum Material Limit | Section 4.1.6 |
| [*_LML] | 0,2,3 | Least Material Limit | Section 4.1.6 |
|  | * | Statistical | Section 4.1.8 |
|  | * | Grundfos Standard Interface | Section 4.1.9 |

* Applies independently of class

Definition of other topics

| Topic | Definition |
| --- | --- |
| CC | Critical Characteristics. Requirements related to CC shall be classified class 2. |
| GSI | Grundfos Standard Interface. Used to control interface of Grundfos preferred module variants. Only for internal Grundfos information. See section 4.1.9 |
| Poka-Yoke | Mistake-proof feature in the product or process. See section 4.1.7 |
| Product Approval | A Product Approval involve a 3rd part or a notified body involvement. Examples of Product Approvals: UL, CSA, VDE, WRAS etc. Relates to Class 0 requirements |
| Product Compliance | A Product Compliance allow self-declaration. Example of Product Compliance: CE, NEC, CED, UBA etc. |
| SCC | Safety Critical Characteristics. Safety Critical Requirements shall be classified class 0 and the relevant hazard type acc. to Section 5.4 shall be applied in bracket after the class 0 symbol. |

Abbreviations used in the standard

| AI | Assembly Instruction acc. to GS401A0020 |
| --- | --- |
| AME | Advanced Manufacturing Engineering |
| A&M | Architecture and Modularisation (department/function in GTI) |
| CI | Control Instruction acc. to GS401A0020 |
| Development | Simplification used to represent “Product Development” |
| GAI | Group Assembly Instruction acc. to GS401A0020 |
| GCI | Group Control Instruction acc. to GS401A0020 |
| GTI | Group Technology & Innovation |
| HAZARD | A Hazard is defined as a potential source of harm |
| HW | Hardware |
| ME | Manufacturing Engineering |
| MECH | Mechanical |
| MEPM | Manufacturing Engineering Project Manager (local). |
| MPM | Manufacturing Product Manager (global). |
| PD | Product Development |
| RISK | Risk is defined as a combination of the probability of occurrence of harm and the severity of the harm |
| SRA | Safety Risk Assessment |

HAZARD types and definition

In the table below the overall HAZARD types used in Grundfos documents are defined. These are used on documents for class 0 Safety Critical Requirements to indicate the type of HAZARD.

Pls. see Figures in Section 4.2.1 which show examples on how to apply the hazard type to different requirements.

| # | Hazard type | Examples |
| --- | --- | --- |
| 1 | Electric shock | - Live parts - Not enough distance to live parts under high voltage - Leakage Current - Short circuit - Electric shock as result of liquid ingress or contamination - Electric shock as result of access to live part Stored Charges - Parts which become live under fault conditions - Electric arcs - All kinds of electrical safety related protection devices (example: Electrical safety switch) - All kinds of information or warning devices - Emergency devices - Warning marks on the product |
| 2 | Thermal/Fire | - Flame - Explosion – Note 1 - Objects or materials with a high or low temperature - Radiation from heat source - Scalding - High pressure fluid ejection/leakage of hot liquid - Chemical burn - All kinds of information or warning devices - Warning marks on the product |
| 3 | Mechanical | - Crushing and cutting - Falling or Ejection of parts - Loss of stability - Hazards resulting from machinery lifting - Hazards caused by missing and/or incorrectly positioned safety related measures/means - All kinds of safeguards - All kinds of safety related protection devices - All kinds of information or warning devices - Emergency devices - Essential equipment and accessories for safe adjusting and/or maintenance - Warning marks on the product - Safe torque feature (on MGE motors) |
| 4 | Contamination | - Drinking water contamination - Biological and microbiological (viral or bacterial) agent |
| 5 | Toxic | - Toxic fluid ejection or leakage |
| 6 | Emission | - Magnetic field (from permanent magnets) - Intended radiation (radio) - Unintended radiation - EMC (Electromagnetic compatibility) - SAR (Specific Absorption Rate) |

Note 1) The Epsilon-x symbol  shall be used on all safety critical requirements on Ex products of the Hazard type “Thermal/fire” where the “Explosion” belongs. Pls. also note procedure for “Design changes of Ex-approved products (ATEX, IECEx or FM)” GP402A0051-01.

Records

No requirements.

References

| Document Number | Document Title (note 1) |
| --- | --- |
| GI318A0001-01 | Classification of drinking water critical requirements |
| GI318A0001-02 | Evaluation of tolerance chains to minimize the need for safety classification |
| GI318A0001-03 | Documentation and classification of critical components in SAP CERTIFICATIONS class |
| GS310A0011 | Technical drawings: Modifiers for dimensional tolerances |
| GS310A0014 | Technical Drawing – stickers for Part/Assembly Drawings and Control/Assembly Instructions. |
| GS 401A0020 | Control and assembly instructions - Definitions and rules |
| GS 402A0003 | Handling of Exemptions |
| GS402A0006 | VPC – Verification of Product & Process Conformance |
| GS402A0043 | Control Plan Standard |
| GS402A0046 | Capability study - Machines, tools and measuring systems |
| GS402A0048 | Failure Mode and Effects Analysis — FMEA |
| GS402A0050 | Determination and calculation of control limits |
| GS402A0051 | Requirements for quality assurance activities for manufacture of products approved according to the ATEX Directive or the IECEx OD/0009 |
| GS402A0068 | Measuring System Qualification (MSQ) and - Documentation (MSD) |
| GS402A0003 | Handling of Exemptions |
| GS402B0002 | Quality Red Alert (RA) – Escalation process |
| ISO 8062-2 | Geometrical product specifications (GPS) – Dimensional and geometrical tolerances for moulded parts – Part 2: Rules |
| ISO 18391 | Geometrical product specifications (GPS) - Population specification |
| GF402A0008-01 | Safety Risk assessment template |

Note 1: These standards are not needed for external partners/suppliers to read and understand 
GS318A0001.

Appendix

Class 0 requirements on Control Instructions

This section specifies how SCC shall be applied on the (Group) Control/Assembly Instruction (CI, GCI, AI and GAI) for Grundfos inhouse produced components/products:

- All SCC requirements on the Part/assembly drawings shall be added to the Control Plan where the measuring, sample frequency and documentation strategy shall be defined

- The SCC requirement shall then be transferred to the relevant CI/GCI/AI/GAI with the control method + samples strategy acc. to GS401A0020

  - The scale 2 class 0 symbol shall be applied in the drawing head

  - The hazard shall not be applied

  - The general SCC sticker shall not be applied

Figure 30: Example showing a Part Drawing (left) with 2 safety critical class 0 requirements (yellow) and one class 0 non-safety requirement (green)together with the corresponding Control Instruction. 
On the Control Instruction (right) the two safety critical requirements (yellow) are shown with class 0 symbol and a control method + samples strategy is defined. The general class 0 sticker and the hazards are not shown. The scale 2 class 0 symbol is also applied to the CI drawing header. In the example the non-safety requirement is not to be controlled as this is only used to document what is tested and approved in the 3rd part approval file.

Legacy definition of Class [1]

| Symbol | Class | Definition |
| --- | --- | --- |
| [1] | 1 | Requirement specified to be class 1. Only to be used on documents released before 1-1-2019 Regulatory requirement without product approval including safety where self-declaration is allowed. No changes are allowed on ratings, designation, composition, geometry, tolerances or supplier of the part without prior acceptance from Development at Grundfos (self-declaration is allowed) Rule regarding change of or exemption to the requirement: The specification cannot be changed, or exemption granted without previous acceptance from the product responsible engineer (PTM/TI/TR). |

- Critical requirements and safety critical requirements which mean that deviation from the given requirements will conflict with regulatory requirements to Product Compliance, such as e.g. the EMC Directive, the Low Voltage Directive, the ATEX Directive, etc.

- The classification shall be applied to all specifications ensuring that regulatory requirements are fulfilled.

- The requirements shall be stated on drawings and when necessary, in related documents, e.g. drawings of raw parts, control instructions, etc. and/or specified in SAP Classification data.

- If the same requirement is related to both class 1 and 0, it shall be classified 0 (the most critical)

- If Grundfos' internal requirements are stricter than the regulatory requirements, class 1 shall still be applied.

- Furthermore, class 1 is applied when a product approval (class 0) has one or more related requirement(s) which do not require acceptance from the approving authority before being changed (Example for Ex approved product: The schedule drawing is class 0 and the related drawings are class 1).

- Example class 1 (both examples may be fulfilled with self-declaration):

  - Guarding of rotational parts related to CE marking acc. to EN809

  - Material composition related to ACS approval

Interpretation
The supplier's or Grundfos' own production equipment and processes together with the accompanying quality assurance activities shall ensure that the requirements for process capability acc. to GS402A0046 are always observed. This implies that Control Limits shall be applied acc. to GS402A0050.

The certification class in SAP shall be updated with the information on which Compliance apply to the part.

| Example | Explanation |
| --- | --- |
|  | Class 1D symbol applied to a geometrical dimension evaluated to be a safety critical characteristic class 1 with documentation. Look in the SAP certification class to see which standard it relates to. |
| Rp02 600MPa min. [1D] | Class 1D requirement applied on a mechanical requirement to ensure a yield strength. |

Application of GS318A0001 to HW documents

Intend for application of GS318A0001 to HW documents:

- Standard Components
Standard components shall not be classified acc. to GS318A0001. Critical Standard components shall be marked critical in SAP certification class.

- PCB layouts – PCB specification
For safety critical PCBs the PCB specification shall be classified according to GS318A0001 by adding a general sticker stating that there are multiple requirements classified. See example Figure 31:

Figure 31; Example of a PCB control board, PCB specification. There are multiple safety critical requirements which is specified by the sticker “Requirements classified acc. to GS 318A0001. Multiple requirements are classified”

- Interface between PCB and Mechanical parts 
This interface shall not be classified acc. to GS318A0001 but evaluated with voltage maps documented in the design journal. MECH and HW interface reviews are carried out (safety distance, creepage)

- Own designed components – transformers, coils etc.
These components shall be classified acc. to GS318A0001

Converting requirements and requirements fulfilled by the chosen process

Converting requirements
A requirement on a Part Drawing or an Assembly Drawing may be converted into process related requirements on the Control Instruction / Control Plan depending on the chosen production process. Under these circumstances the capability on the translated process related requirements shall be demonstrated during daily operation.

Example:     Coaxiality of a cylindrical feature may be converted into regular inspections of run-out and inspections of roundness with longer intervals between the inspections.

Requirements fulfilled by smart processing/fixturing
Furthermore, some of the requirements may be fulfilled by choosing process and fixture. For such requirements it is recommended to show conformance with a capability study during the VPC and during the running production from time to time measure the requirement.

Example:     Coaxiality of two cylindrical features which can be ensured by machining in the same fixture.

For internal Grundfos processes, the correlation between requirement on Part/Assembly Drawing and Control Instruction shall be documented in the measuring system documentation (MSD) acc. to GS402A0068.

With regards to products from external suppliers, the correlation shall appear from the supplier's quality assurance documentation, typically documented in the Control Plan, which shall be reviewed and approved by the SQA engineer.

Note – double meaning of the class 0 symbol: ISO 8062-2 vs. GS318A0001

If you use graphical symbols acc. to ISO 8062 then please be aware that there is a risk to have double meaning of the class 0:

Both the GS318A0001 and ISO 8062-2 (Geometrical Product Specification (GPS) – Dimensional and geometrical tolerances for moulded parts – Part 2: Rules) use the triangle in a circle, but with different definition:

| Standard | Symbol | Definition | Explanation |
| --- | --- | --- | --- |
| Grundfos Standard GS318A0001 |  | Classification Class 0 – Critical regarding regulatory requirements with product approval. | Added as classification to requirement to indicate that the requirement is related to a regulatory requirement and cannot be changed without previous acceptance from the approving authority. |
| ISO 8062-2 |  | Drawing identifier for final machined moulded part | Used on combined drawings of casted parts where the raw and machined part is shown in the same drawing. The symbol indicates that the requirement applies in machined condition. |
| Conclusion Grundfos does not use combined drawings of moulded/casted parts: We have 2-3 drawing and BOM levels: Raw, Machined and for some parts also Coated (CED/paint), each with their own drawing and requirements. Grundfos always reference the GS318A0001 in the vicinity of the drawing head when we use classification symbols. Due to this there is no risk with this double definition of the symbol. Furthermore, Grundfos has used the symbol for 20+ years and have a huge number of drawings using the symbol (update with a new class 0 symbol would require huge effort). |  |  |  |